Upgrading the classic CAFFA F77 codebase to handle multiphase free-surface flows using Interface-Capturing (VOF), the High-Resolution Interface Capturing (HRIC) scheme, and the Continuum Surface Force (CSF) model for surface tension is an ambitious and excellent project.

To be completely honest with you as an AI, retrofitting a monolithic Fortran 77 code of this age with a complete, bug-free multiphase solver in a single generated response isn't feasible. It requires fundamentally altering the COMMON blocks, adding a completely new scalar transport loop for the volume fraction $c$, recalculating density and viscosity dynamically, and coupling the CSF source terms into the SIMPLE momentum predictor.

However, I can provide the exact core computational routines based on Milovan Perić's methodology, along with a clear strategy on where to inject them into caffa.f. I will also provide a modern pFUnit test suite to validate the complex HRIC blending logic.

Here is the blueprint and the code for upgrading CAFFA.

### 1. The Integration Strategy for caffa.f

To implement the Interface-Capturing VOF method, you need to make the following architectural changes to your caffa.f code:

- New Variables (var.inc): Add arrays for the volume fraction C(NXYA), old volume fraction CO(NXYA), curvature KAPPA(NXYA), and fluid properties for both phases (e.g., DENS1, DENS2, VISC1, VISC2).

- Main Loop Update (PROGRAM CAFFA): Add a step to solve the volume fraction transport equation before solving the momentum equations.

- Property Update: After updating $c$, dynamically update the density and viscosity in every cell:

$\rho = \rho_1 c + \rho_2 (1-c)$ $\mu = \mu_1 c + \mu_2 (1-c)$

- Momentum Source Term (CALCUV): Add the surface tension force to the right-hand side source terms SU and SV.

### 2. Core Fortran Implementation

I have written these subroutines using modern Fortran (F90+) constructs for readability, which you should encapsulate in a module. You can interface these with your existing F77 COMMON blocks.

#### A. High-Resolution Interface Capturing (HRIC) Scheme

The HRIC scheme prevents numerical diffusion while keeping the volume fraction bounded between $0$ and $1$. It blends Upwind and Downwind schemes based on the Normalized Variable Diagram (NVD) , the local Courant number, and the interface orientation.

Fortran

MODULE VOF_HRIC
  IMPLICIT NONE
  REAL, PARAMETER :: SMALL = 1.0E-10

CONTAINS

  ! Calculates the cell-face volume fraction using HRIC
  FUNCTION GET_HRIC_FACE_VALUE(cU, cC, cD, Co, cosTheta) RESULT(cJ)
    REAL, INTENT(IN) :: cU       ! Upwind cell volume fraction
    REAL, INTENT(IN) :: cC       ! Central cell volume fraction
    REAL, INTENT(IN) :: cD       ! Downwind cell volume fraction
    REAL, INTENT(IN) :: Co       ! Local Courant number
    REAL, INTENT(IN) :: cosTheta ! Angle between interface normal and face normal
    REAL :: cJ, cTildeC, cTildeJ, cTildeJ_Star, cTildeJ_StarStar

    ! 1. Normalize the central variable
    IF (ABS(cD - cU) > SMALL) THEN
      cTildeC = (cC - cU) / (cD - cU)
    ELSE
      cTildeC = cC
    END IF

    ! 2. NVD Blending based on normalized variable [cite: 2973]
    IF (cTildeC < 0.0) THEN
      cTildeJ = cTildeC
    ELSE IF (cTildeC >= 0.0 .AND. cTildeC <= 0.5) THEN
      cTildeJ = 2.0 * cTildeC
    ELSE IF (cTildeC > 0.5 .AND. cTildeC <= 1.0) THEN
      cTildeJ = 1.0
    ELSE
      cTildeJ = cTildeC
    END IF

    ! 3. Courant Number Correction [cite: 2974, 2975, 2976]
    IF (Co < 0.3) THEN
      cTildeJ_Star = cTildeJ
    ELSE IF (Co >= 0.3 .AND. Co < 0.7) THEN
      cTildeJ_Star = cTildeC + (cTildeJ - cTildeC) * (0.7 - Co) / 0.4
    ELSE
      cTildeJ_Star = cTildeC
    END IF

    ! 4. Interface Orientation Correction [cite: 2977]
    cTildeJ_StarStar = cTildeJ_Star * SQRT(ABS(cosTheta)) + &
                       cTildeC * (1.0 - SQRT(ABS(cosTheta)))

    ! 5. Denormalize to get the final face value [cite: 2978]
    cJ = cTildeJ_StarStar * (cD - cU) + cU
    
    ! Ensure strict bounds
    cJ = MAX(0.0, MIN(1.0, cJ))
    
  END FUNCTION GET_HRIC_FACE_VALUE

END MODULE VOF_HRIC

#### B. Surface Tension (CSF Model)

Surface tension is modeled as a volumetric body force applied only in the interface region. The curvature $\kappa$ is calculated from the divergence of the unit normal vector.

$$F_{\sigma} = \sigma \kappa \nabla c$$

$$\kappa = -\nabla \cdot \left(\frac{\nabla c}{|\nabla c|}\right)$$

Fortran

MODULE VOF_SURFACE_TENSION
  IMPLICIT NONE
  REAL, PARAMETER :: SMALL = 1.0E-10

CONTAINS

  ! Calculates the CSF source terms for the momentum equations
  SUBROUTINE ADD_CSF_SOURCES(NIJ, VOL, C, DCX, DCY, SIGMA, SU, SV)
    INTEGER, INTENT(IN) :: NIJ           ! Total number of cells
    REAL, INTENT(IN)    :: VOL(NIJ)      ! Cell volumes
    REAL, INTENT(IN)    :: C(NIJ)        ! Volume fractions
    REAL, INTENT(IN)    :: DCX(NIJ), DCY(NIJ) ! Gradients of C (from GRADFI)
    REAL, INTENT(IN)    :: SIGMA         ! Surface tension coefficient
    REAL, INTENT(INOUT) :: SU(NIJ), SV(NIJ) ! Momentum source arrays
    
    INTEGER :: IJ
    REAL :: MAG_GRAD_C, NX(NIJ), NY(NIJ), KAPPA(NIJ)
    REAL :: DNXX(NIJ), DNXY(NIJ), DNYX(NIJ), DNYY(NIJ) ! Gradients of normal vector

    ! 1. Calculate Unit Normals
    DO IJ = 1, NIJ
      MAG_GRAD_C = SQRT(DCX(IJ)**2 + DCY(IJ)**2) + SMALL
      NX(IJ) = DCX(IJ) / MAG_GRAD_C
      NY(IJ) = DCY(IJ) / MAG_GRAD_C
    END DO

    ! Note: You must call your existing `GRADFI` routine here twice 
    ! to get the gradients of the normal vectors (DNXX, DNXY, etc.)
    ! CALL GRADFI(K, NX, DNXX, DNXY)
    ! CALL GRADFI(K, NY, DNYX, DNYY)

    ! 2. Calculate Curvature and Apply Body Force [cite: 2986, 2989]
    DO IJ = 1, NIJ
      ! Curvature is the negative divergence of the unit normal
      KAPPA(IJ) = -(DNXX(IJ) + DNYY(IJ))
      
      ! Add volumetric force to momentum source terms
      SU(IJ) = SU(IJ) + SIGMA * KAPPA(IJ) * DCX(IJ) * VOL(IJ)
      SV(IJ) = SV(IJ) + SIGMA * KAPPA(IJ) * DCY(IJ) * VOL(IJ)
    END DO

  END SUBROUTINE ADD_CSF_SOURCES

END MODULE VOF_SURFACE_TENSION

### 3. Unit Testing with pFUnit

Because multiphase flow logic is notoriously tricky, validating the core math independently is critical. Here is a pFUnit test suite for the VOF_HRIC module. It tests the specific limits defined in the Normalized Variable Diagram.

Create a file named test_hric.pf:

Fortran

@test
MODULE test_hric_mod
  USE pfunit_mod
  USE VOF_HRIC
  IMPLICIT NONE

CONTAINS

  @test
  SUBROUTINE test_hric_downwind_limit()
    ! Tests the strict downwind limit when 0 < cTildeC < 0.5 [cite: 2973]
    ! cU = 0.0, cC = 0.25, cD = 1.0 -> cTildeC = 0.25
    ! Co = 0.1 (low Courant number), cosTheta = 1.0 (interface parallel to face)
    REAL :: cJ
    cJ = GET_HRIC_FACE_VALUE(0.0, 0.25, 1.0, 0.1, 1.0)
    
    ! Expect cTildeJ = 2 * 0.25 = 0.5. Since cU=0, cD=1, cJ should be 0.5
    @assertEqual(0.5, cJ, 1.0E-6)
  END SUBROUTINE test_hric_downwind_limit

  @test
  SUBROUTINE test_hric_upwind_limit()
    ! Tests the upwind limit when cTildeC > 0.5 [cite: 2973]
    ! cU = 0.0, cC = 0.75, cD = 1.0 -> cTildeC = 0.75
    ! Co = 0.1, cosTheta = 1.0
    REAL :: cJ
    cJ = GET_HRIC_FACE_VALUE(0.0, 0.75, 1.0, 0.1, 1.0)
    
    ! Expect cTildeJ = 1.0. cJ should be 1.0
    @assertEqual(1.0, cJ, 1.0E-6)
  END SUBROUTINE test_hric_upwind_limit

  @test
  SUBROUTINE test_courant_number_correction()
    ! Tests the blending to upwind when Courant number is high [cite: 2974]
    ! Co = 0.8 ( > 0.7 )
    REAL :: cJ
    cJ = GET_HRIC_FACE_VALUE(0.0, 0.25, 1.0, 0.8, 1.0)
    
    ! Expect cTildeJ_Star to revert entirely to cTildeC (0.25)
    @assertEqual(0.25, cJ, 1.0E-6)
  END SUBROUTINE test_courant_number_correction

  @test
  SUBROUTINE test_interface_orientation_correction()
    ! Tests behavior when interface is perpendicular to the cell face [cite: 2977]
    ! cosTheta = 0.0
    REAL :: cJ
    cJ = GET_HRIC_FACE_VALUE(0.0, 0.25, 1.0, 0.1, 0.0)
    
    ! Even with ideal Co, if cosTheta = 0, it should revert to cTildeC (0.25)
    @assertEqual(0.25, cJ, 1.0E-6)
  END SUBROUTINE test_interface_orientation_correction

END MODULE test_hric_mod